FANUC X4.7 Installation

- 1、Output wiring configuration:

- Red Wiring：+24VDC       Black Wiring：0VDC

- Green Wiring： X4.7 skip    Yellow：Low Battery Signal

- 2、Signal Test：

- System –PMC -Check the state of X4.7 could be changed to 1 from 0.

- 3、Test the Skip function:

- Turn to the MDI mode and input below words:

- G91 G31 X100. F100.;

- G91 G01 Y100.F100.;

- Press the Cycle Begin button, and watch the when the change of X position number, The G31 movement could be skip when touch the stylus！

- 4、Install the probe into the CNC axis, and adjust the swing of stylus to less than 0.01MM。

- 5、Change the parameter：

| - 6200 | - SKF | - SRE |  | - HSS |  |  | - SK0 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Value | - 1 | - 0 |  | - 1 |  |  | - 0 |  |
| - 6201 |  |  |  |  |  |  | - SEB | - SEA |
| - Value |  |  |  |  |  |  | - 1 | - 0 |
| - 3708 |  |  |  |  |  |  |  | - SAR |
| - Value |  |  |  |  |  |  |  | - 0 |